A.R.E.N.A. VIETNAM — QUALITY & COMPLIANCE

Master Quality Compliance Guideline

Manual Hand Pallet Truck (pedestrian-propelled, hydraulic lift)

| Document reference | MCG-DIY-HT-PT-001 |
| --- | --- |
| Version / status | v1.0 — Draft for CQAD approval |
| Issue date | 14/10/2026 |
| Department | DIY |
| Product Family (QSD level) | Hand Tools |
| Product Sub-Family (QCP level) | Storage / Moving |
| Compliance pathway | PATHWAY A — CE marking · Machinery Directive 2006/42/EC → Machinery Regulation (EU) 2023/1230 from 20/01/2027 · Internal production control (Module A), no Notified Body |
| Target markets | EU-27 + Switzerland |
| Allies | HAGEBAU · ITM · BCM · DEDEMAN · JUMBO/Coop |
| Mode | Mode 1 — Category level (supplier / factory fields left as [___]) |
| Source document | Product requirements – pallet truck (sample validation checklist, A.R.E.N.A. template) |
| Prepared by / Approved by | QS [___] / CQAD [___] |

Disclaimer: standard editions, clause numbers and OJEU citation status marked “verify” must be confirmed against the current official texts before this guideline is released as controlled document.

Contents

(Right-click → Update field in Word to refresh page numbers.)

# 1. Executive Summary

- Pathway A (CE). A manual pallet truck is lifting machinery whose only power source is directly applied human effort — within the scope of the Machinery Directive 2006/42/EC (Art. 2(a), 4th indent) and of the Machinery Regulation (EU) 2023/1230 from 20/01/2027.

- Regulatory switch-over. Units placed on the EU market up to 19/01/2027 → MD 2006/42/EC. Units placed on the market from 20/01/2027 → MR (EU) 2023/1230 (new DoC wording, Annex III EHSR, importer marking, instructions rules). Any P.O. with EU arrival on or after 20/01/2027 shall be documented under the MR.

- Conformity assessment. Not listed in MD Annex IV / MR Annex I → internal production control (MD Art. 12(2), Annex VIII; MR Module A, Annex VI). No Notified Body; A.R.E.N.A. requires an ISO/IEC 17025-accredited third-party test report.

- Harmonised standards. EN ISO 3691-5:2015 + A1:2020 (pedestrian-propelled trucks), EN 16307-5:2013 (supplementary requirements), EN ISO 12100:2010 (risk assessment). Verify OJEU citation under the MR.

- Horizontal overlays. REACH (Art. 33 SVHC, Annex XVII entries 50 PAH and 51 phthalates for grips/wheels), POPs (EU) 2019/1021 (SCCP), PPWR (EU) 2025/40 (applicable since 12/08/2026), GPSR (EU) 2023/988 (complementary), Reg. (EU) 2019/1020 Art. 4 (EU economic operator).

- Key corrections to the source checklist: GPSD 2001/95/EC → GPSR; PPWD 94/62/EC → PPWR; dynamic test (1.1 × rated load), load holding, overload device, crushing points, data plate and instructions checks added; AQL sampling plan defined. Full log in Section 14.

- Release (SRC) critical items: valid test report covering the variant, signed DoC in Ally language(s), CE + complete data plate incl. rated capacity, paper instructions in the destination language(s), FRI PASS (Cr 0 / Ma 2.5 / Mi 4.0), PPWR packaging data, SVHC declaration.

# 2. Link 1 — 4-Step Compliance Determination

## 2.1 Step I — Intended use and foreseeable misuse

| Function | Horizontal transport of palletised loads up to the rated capacity Q (typically 2 000–3 000 kg at load centre c = 600 mm — confirm per offer); fork lift by pumping the tiller; controlled lowering by hand lever (and/or foot pedal). |
| --- | --- |
| Operating environment | Smooth, level, hard floors; indoor warehouses, shops, workshops (EN ISO 3691-5 Cl. 3 definition — verify). |
| User population | Professional operators and non-professional DIY customers (DIY retail channel → consumer use assumed for instructions and safety information). |
| Reasonably foreseeable misuse | Overloading; eccentric / off-centre loads; use on slopes or uneven ground; riding on the truck; lifting persons; use as vehicle jack; parking on gradient without brake; children playing with parked truck; tiller released under load. |
| Vehicle component? | No → Pathway D excluded |
| Biocidal claim? | No → Pathway E excluded |

## 2.2 Step II — Directive scope check

| Directive / Regulation | Scope clause | In scope? | Justification |
| --- | --- | --- | --- |
| Machinery Directive 2006/42/EC | Art. 1(1)(a); Art. 2(a) 4th indent | YES (until 19/01/2027) | Assembly for lifting loads whose only power source is directly applied human effort. |
| Machinery Regulation (EU) 2023/1230 | Art. 2; Art. 3 definition of “machinery” (manual lifting assemblies) — verify point | YES (from 20/01/2027) | Same scope logic carried over from MD. |
| LVD 2014/35/EU / EMC 2014/30/EU | Art. 1 / Art. 2 | NO | No electrical equipment. |
| RED 2014/53/EU / RoHS 2011/65/EU | Art. 1 / Art. 2 | NO | No radio or EEE. |
| Toys Reg. (EU) 2025/2509 | Art. 2 | NO | Not designed for play. |
| PPE (EU) 2016/425 / CPR 305/2011 | Art. 3 / Art. 2 | NO | Not PPE; not incorporated in construction works. |
| GPSR (EU) 2023/988 | Art. 2(1) 2nd subpara. | COMPLEMENTARY | Applies to aspects/risks not covered by MD/MR and to obligations such as accident notification (Art. 20), online sales (Art. 22) and recalls (Art. 35–37). |
| Market Surveillance Reg. (EU) 2019/1020 | Art. 4 | YES | Machinery listed in Art. 4(5): an EU-established economic operator is mandatory; name and contact on product, packaging or accompanying document. |

## 2.3 Step III — Material composition overlays

| Material / signal | Overlay | Requirement / action |
| --- | --- | --- |
| All articles | REACH Art. 33 | SVHC declaration per shipment (Candidate List current at shipment date, > 0.1 % w/w per article). |
| Tiller grip, wheels (PU/rubber/PVC) — skin contact | REACH Annex XVII entry 50 | 8 PAH ≤ 1 mg/kg each (articles supplied to general public, prolonged/repetitive skin contact). |
| Plasticised PVC grip / hoses | REACH Annex XVII entry 51 | DEHP, DBP, BBP, DIBP ≤ 0.1 % (sum) in plasticised material. |
| PVC / rubber parts | POPs Reg. (EU) 2019/1021 | SCCP ≤ 1 500 mg/kg (0.15 %) in articles — verify current limit. |
| Hydraulic oil (contained in pump) | REACH / CLP | Oil is part of the article; provide SDS of the oil type for maintenance; oil type and volume in instructions. |
| Painted / powder-coated steel | REACH SVHC (Pb chromates) | Lead chromate pigments are SVHC/authorisation substances — confirm lead-free coating. |
| Steel frame / forks | CBAM (EU) 2023/956 | CN 8427 90 00 not in CBAM Annex I → out of scope (verify CN code on P.O.). |
| Wooden pallet / crate | ISPM 15 · EUDR (EU) 2023/1115 | HT-treated with IPPC mark. Packaging used solely to support/protect another product is excluded from EUDR (Annex I). |
| All packaging | PPWR (EU) 2025/40 | Art. 5(4): Pb+Cd+Hg+Cr(VI) ≤ 100 mg/kg; packaging data per SKU; EPR registration (see Link 8). |

## 2.4 Step IV — Market destination

| Market / Ally | Parallel requirements activated |
| --- | --- |
| EU-27 (all) | MD/MR + CE; EU economic operator (2019/1020 Art. 4); GPSR complementary obligations; REACH + POPs; PPWR + national EPR; instructions and DoC in official language(s) of the Member State of sale. |
| Switzerland — JUMBO/Coop | Machinery Ordinance (MaschV, SR 819.14) — EU-CH MRA recognises EU conformity; Product Safety Act (PrSG, SR 930.11); instructions DE/FR/IT; Swiss distributor/importer identification. Monitor Swiss alignment to MR (EU) 2023/1230. |
| France — ITM | French language (Loi Toubon); Triman + Info-tri sorting information (Code de l’environnement L.541-9-3); mineral oils in packaging inks — Law 2020-105 Art. 112 and implementing decree. |
| Germany — HAGEBAU | German language; GS mark (ProdSG §20–22) only if required by Ally — then AfPS GS 2019:01 PAK applies to grips. |
| Romania — DEDEMAN | Romanian language on instructions, warnings and DoC. |
| BCM | Language(s) and EPR per destination country — [confirm with Ally]. |

### Gate 1 decision

| ✅ GO | Pathway A assigned with scope-clause justification (this section) — current status: GO |
| --- | --- |
| ⚠️ CONDITIONAL | Truck variant with electric lift/travel or scale (EEE) → re-run Link 1 (LVD/EMC/RoHS/WEEE/batteries). |
| ❌ NO-GO | Rated capacity, lift height or variant list not defined by supplier. |

# 3. Link 2 — Pathway Assignment and Directive Stack

| Directive / Regulation | Scope | hEN / reference | Pathway | Module | NB? | Declaration |
| --- | --- | --- | --- | --- | --- | --- |
| MD 2006/42/EC | Yes — until 19/01/2027 | EN ISO 3691-5:2015+A1:2020; EN 16307-5:2013; EN ISO 12100:2010 | A | Annex VIII (internal) | No | EC DoC (Annex II 1.A) |
| MR (EU) 2023/1230 | Yes — from 20/01/2027 | Standards cited under MR — monitor OJEU; until cited, apply above as state of the art and document new EHSR gaps | A (B if no citation) | Module A (Annex VI) | No | EU DoC (Annex V) |
| GPSR (EU) 2023/988 | Complementary | — | — | — | No | None (safety information, traceability) |
| Reg. (EU) 2019/1020 | Art. 4 | — | — | — | No | EU operator identification |
| REACH / POPs | Overlay | Annex XVII; Candidate List | — | — | No | SVHC declaration + test report |
| PPWR (EU) 2025/40 | Overlay | Art. 5, 12, 24, 38–39 (verify application dates per article) | — | — | No | Packaging conformity documentation (packaging manufacturer) |

### Gate 2 decision

| ✅ GO | Single pathway A; directive stack complete — current status: GO |
| --- | --- |
| ⚠️ CONDITIONAL | P.O. delivery straddles 20/01/2027 → prepare dual MD/MR documentation. |
| ❌ NO-GO | Pathway cannot be confirmed → return to Link 1. |

# 4. Link 3 — Module Determination

| Directive | Pathway | Module | NB required | NB name / no. | Certificate ref. |
| --- | --- | --- | --- | --- | --- |
| MD 2006/42/EC | A | Internal production control — Art. 12(2), Annex VIII | No | N/A | N/A |
| MR (EU) 2023/1230 | A | Module A — internal production control, Annex VI | No | N/A | N/A |

Rationale: manual pallet trucks are not listed in MD Annex IV nor in MR Annex I (Parts A/B). The module is prescribed by the legislation, not chosen by the manufacturer. A.R.E.N.A. requirement: conformity evidence by ISO/IEC 17025-accredited third-party laboratory test report against the hEN(s); voluntary NB/certification body attestation is accepted but does not alter the module. No NB number follows the CE marking.

### Gate 3 decision

| ✅ GO | Module confirmed; no NB required — current status: GO |
| --- | --- |
| ⚠️ CONDITIONAL | Accredited lab not yet appointed — engagement plan before sample validation. |
| ❌ NO-GO | Supplier declares NB number on marking/DoC without NB involvement (false NB reference). |

# 5. Link 4 — Type Testing and Technical File

## 5.1 Type-test matrix (ITT-equivalent for Module A)

| # | Test / characteristic | Reference (verify clause) | Requirement |
| --- | --- | --- | --- |
| 1 | Static test | MD/MR Annex I/III 4.1.2.3, 4.1.3; EN ISO 3691-5 | 1.5 × Q at load centre c; no permanent deformation, no patent defect, no leakage. |
| 2 | Dynamic test | MD/MR 4.1.2.3, 4.1.3 | 1.1 × Q; all functions (lift, lower, travel, steer) operate correctly. |
| 3 | Overload / pressure-limiting device | EN ISO 3691-5 Cl. 4.4.3.3 (per source) | Set < 115 % of maximum working pressure; cannot work loose; tool/key needed to alter. Manual trucks: design shall allow fitting. A.R.E.N.A./Ally: device fitted — [confirm]. |
| 4 | Stability | EN ISO 3691-5 / EN 16307-5 | Stability verification per standard (tilt-table or calculation). |
| 5 | Lifting / lowering control | MD/MR 1.2.1, 1.2.2, 4.1.2.6; EN ISO 3691-5 | Controls identifiable; controlled lowering under Q; no uncontrolled descent; lowering-speed limit per standard. |
| 6 | Load holding (drift) | EN ISO 3691-5 | Drift of raised loaded forks within the limit of the standard over the specified time. |
| 7 | Tiller and hand grips | EN ISO 3691-5 Cl. 4.2.2 (per source) | Closed-loop handle or lateral hand protection; grip cross-section within 25 mm / 35 mm concentric circles; span ≥ 120 mm per hand; operating forces per standard. |
| 8 | Crushing / shearing points, feet protection | MD/MR 1.3.7, 1.3.8; EN ISO 13854:2019; EN ISO 3691-5 | Gaps at tiller pivot, linkage, fork rollers: fingers ≥ 25 mm or ≤ safe gap; steer-wheel feet protection per standard. |
| 9 | Edges and surfaces | MD/MR 1.3.4 | No sharp edges, angles or rough surfaces on accessible parts. |
| 10 | Parking brake (if fitted) | EN ISO 3691-5 | Holds laden truck on gradient specified by the standard / instructions. |
| 11 | Marking and instructions | MD/MR 1.7.3, 1.7.4, 4.3.3, 4.4.2; EN ISO 3691-5 Cl. 7 | Content check — see Link 7. |
| 12 | Coating performance | ISO 2409; ISO 9227 NSS; ISO 2808 | Cross-cut ≤ class 2; salt-spray hours and DFT per QSD/Ally spec [___]. |
| 13 | Chemical suite | REACH Annex XVII 50, 51; Art. 33; POPs | PAH, phthalates (grips/wheels), SCCP, SVHC screening. Validity 2 years. |
| 14 | Packaging heavy metals | PPWR Art. 5(4) | Pb+Cd+Hg+Cr(VI) ≤ 100 mg/kg per packaging component. |

Variant coverage: one report per capacity / fork length / fork width configuration, or documented worst-case justification in the technical file. Re-test on any change of frame, fork section, pump, seals, wheel material, coating system or production site.

## 5.2 Technical file content (MD Annex VII A / MR Annex IV)

- General description; overall drawing; detailed drawings of frame, forks, pump and tiller; hydraulic schematic and calculation notes (strength, stability).

- Risk assessment per EN ISO 12100: hazards, protective measures, residual risks (covers all misuse in 2.1).

- List of EHSR applied and harmonised standards with edition dates; justification for any deviation.

- Test reports (Section 5.1) and end-of-line test records.

- Instructions for use (all languages) and copy of DoC.

- Internal measures ensuring series conformity (FPC — Link 5).

- Retention: 10 years from the last unit placed on the market; available to authorities on reasoned request.

### Gate 4 decision

| ✅ GO | Accredited test report covering the exact variant, current hEN editions, all 14 items addressed; technical file complete. |
| --- | --- |
| ⚠️ CONDITIONAL | Chemical report expiring < 3 months; MR gap analysis pending for deliveries after 20/01/2027. |
| ❌ NO-GO | Static/dynamic test missing or failed; report covers a different model; design changed since test. |

# 6. Link 5 — Factory Production Control and Inspection

## 6.1 Factory prerequisites (from source, updated)

| Criteria | Requirement |
| --- | --- |
| Factory QMS | A.R.E.N.A. QMS audit grade A/B (C = conditional with CAPA). Valid ISO 9001 certificate is considered satisfying the QMS requirement. FPC audit confirming series conformity to the tested type. |
| Factory CSR | amfori BSCI A/B/C (C = conditional). Valid SA8000 is considered satisfying. ISO 14001 where applicable. |

## 6.2 FPC critical control points

| Component / process | Control parameter | Frequency / record |
| --- | --- | --- |
| Steel (frame, forks) | Grade and thickness vs drawing; mill certificate | Each heat / lot |
| Welding | WPS; welder qualification; visual weld inspection (no cracks, porosity, undercut) | Each shift; 100 % visual on load-bearing welds |
| Hydraulic pump | 100 % pressure/leak test; overload valve setting and seal; oil type and volume | 100 %, recorded |
| End-of-line load test | Lift / hold / lower with test load per MD/MR 4.1.3 (static and dynamic tests on lifting machinery) | 100 % or documented sampling justified in risk assessment — verify against Guide to MD §§ on 4.1.3 |
| Wheels and bearings | Material (PU/nylon), diameter, free rotation | Each lot |
| Fasteners, pins, circlips | Presence, torque, securing | 100 % visual |
| Coating | DFT (ISO 2808), cross-cut (ISO 2409) | Each batch |
| Marking | Data plate content, serial number, CE, rated capacity | 100 % |
| Non-conforming product / CAPA / calibration | Segregation, disposition, 8D, gauge calibration | Per procedure; records ≥ 10 years |

## 6.3 A.R.E.N.A. inspection schedule

| Stage | Trigger | Quantity / sampling | Checklist |
| --- | --- | --- | --- |
| Sample validation | New item / new supplier / change | 1 pc (+1 pc golden sample retained) | Annex A — all items incl. static test |
| DPI / in-line | First order; critical order | At ≥ 20 % of goods produced | Annex A items 1–7 (static test on 1 pc) |
| FRI | Every shipment, mandatory | 100 % produced, ≥ 80 % packed; ISO 2859-1 / ANSI Z1.4 GI-II; Cr 0 · Ma 2.5 · Mi 4.0; functional tests at S-3 | Annex A (static test not required unless Ally requests) |
| Golden sample | Every P.O., each variant | Sealed reference | Per QSD |

### Gate 5 decision

| ✅ GO | FPC verified; sample validation PASS; FRI booked. |
| --- | --- |
| ⚠️ CONDITIONAL | First order — DPI mandatory before FRI. |
| ❌ NO-GO | No end-of-line load test; FRI critical defect; FPC not established. |

# 7. Link 6 — Declaration of Conformity

| DoC element | MD 2006/42/EC — Annex II 1.A | MR (EU) 2023/1230 — Annex V |
| --- | --- | --- |
| Manufacturer name and full address | Required (+ authorised representative) | Required (+ authorised representative) |
| Person authorised to compile technical file (established in EU) | Required | Not a DoC field — verify |
| Description and identification | Generic denomination, function, model, type, serial number, commercial name | Product, model, type, serial/batch; image optional |
| Sole responsibility statement | — | Required |
| Conformity statement | Machinery Directive + other applicable directives | MR + other Union harmonisation legislation |
| Standards | Harmonised standards with dates | Harmonised standards / common specifications with dates |
| NB | N/A (Annex VIII) | N/A (Module A) |
| Place, date, name, function and signature | Required | Required |

- One DoC per model; languages of each Member State of sale (EN + DE/FR/IT/RO etc.).

- MD: the instructions shall contain the DoC or a document setting out its contents (Annex I 1.7.4.2(c)). MR: DoC accompanies the product or the instructions give its internet address — verify MR article before release.

- Superseded references (e.g. EN 1757-2, GPSD 2001/95/EC) on DoC → reject.

### Gate 6 decision

| ✅ GO | DoC complete, signed, correct legal act for the placing-on-market date, correct languages. |
| --- | --- |
| ⚠️ CONDITIONAL | Cites superseded standard edition — revise before shipment. |
| ❌ NO-GO | DoC missing/unsigned; MD DoC used for units placed on market after 19/01/2027. |

# 8. Link 7 — CE Marking, Data Plate and Instructions

## 8.1 Marking verification checklist (QC at FRI)

| Element | Reference | Accept criterion | Fail class |
| --- | --- | --- | --- |
| CE marking ≥ 5 mm, Annex II 765/2008 proportions, indelible, next to manufacturer name | MD 1.7.3 / Art. 16; MR Art. 24 | Present, correct, no NB number | Critical |
| Manufacturer business name and full address (+ AR) | MD/MR 1.7.3 | Complete | Critical |
| Importer name, trade name, postal address and contact | MR Art. 13; GPSR; 2019/1020 Art. 4 | On product or, where impossible, packaging/document | Critical |
| Designation, series or type, serial number, year of construction | MD/MR 1.7.3 | Complete and legible | Critical |
| Rated capacity (kg) at load centre (mm); unladen mass | MD/MR 4.3.3; EN ISO 3691-5 data plate (verify) | Matches test report and spec | Critical |
| Warning pictograms (no riding, foot crushing, max load) | Risk assessment; EN ISO 3691-5 | Present, durable | Major |
| Swiss distributor identification (CH lots) | PrSG | Present | Major |
| Barcode / EAN | Ally spec | Scans correctly, position per approval | Major |
| Triman + Info-tri (FR lots) | L.541-9-3 C. env. | On packaging or leaflet | Major |

## 8.2 Instructions for use — minimum content

- Intended use, rated capacity, load centre, permitted floors; prohibited uses (2.1 misuse list).

- Assembly of tiller (pin, chain/rod adjustment), commissioning check, operation (lift, neutral, lower), parking/brake.

- Maintenance: oil type, volume, refilling, air bleeding, lubrication points, inspection intervals, wear limits; residual risks.

- Manufacturer and importer identification; copy/content of DoC (MD) or DoC access (MR).

- Languages: official language(s) of each country of sale (CH: DE/FR/IT). Paper format — required in practice for the DIY channel; under the MR digital instructions are allowed, but safety information for non-professional users must be on paper and a paper copy supplied free on request (verify MR Art. 10(7)).

- Layout per EN IEC/IEEE 82079-1 (assessment via assembly-instruction-assessment skill).

### Gate 7 decision

| ✅ GO | All Critical marking elements present; instructions approved in all languages. |
| --- | --- |
| ⚠️ CONDITIONAL | Minor label error correctable before carton sealing. |
| ❌ NO-GO | CE/data plate/rated capacity/importer missing; wrong language; NB number without NB. |

# 9. Link 8 — SRC Release Conditions

- Links 1–3: MCG on file; pathway and module confirmed; legal act matches EU arrival date (MD ≤ 19/01/2027 / MR ≥ 20/01/2027).

- Link 4: accredited test report valid and covering every variant on the P.O.; chemical report ≤ 2 years.

- Link 5: FRI PASS — Cr 0 / Ma 2.5 / Mi 4.0; functional tests PASS; golden sample match; DPI PASS for first order; end-of-line load-test records sighted.

- Link 6: signed DoC in required languages on file.

- Link 7: marking and instructions verified at FRI.

- PPWR: packaging weight and material per SKU recorded (Annex D); heavy-metal compliance; EPR registration of the producer for each destination country.

- REACH: SVHC declaration for the shipment on file.

- Wooden pallets: ISPM 15 HT mark verified.

| Ally / market | Additional release condition |
| --- | --- |
| HAGEBAU (DE) | German instructions/DoC; GS certificate valid if GS ordered. |
| ITM (FR) | French instructions; Triman/Info-tri; mineral-oil-free packaging inks declaration. |
| DEDEMAN (RO) | Romanian instructions/DoC. |
| JUMBO/Coop (CH) | DE/FR/IT instructions; Swiss distributor identification. |
| BCM | Destination languages and EPR — [confirm]. |

SRC decision: RELEASED (all gates PASS) · RELEASED WITH CONDITIONS (minor items closed before vessel departure) · HOLD (any critical gate FAIL; escalate to QS Manager and notify Ally if sailing window impacted). Signed by QS; QC countersignature on FRI input.

# 10. Document Register

| # | Document | Link | Issued by | Validity | Retention |
| --- | --- | --- | --- | --- | --- |
| 1 | Quality Compliance Pathway (QCP) — Storage/Moving | 1–2 | QS | On change | 10 yr |
| 2 | Quality Single Document (QSD) — Hand Tools | 1–6 | QS | On change | 10 yr |
| 3 | Harmonised standard lookup record (MD + MR) | 1–3 | QS | On OJEU change | 10 yr |
| 4 | Risk assessment EN ISO 12100 / DFMEA | 4 | Manufacturer / QS | On change | 10 yr |
| 5 | Type test report EN ISO 3691-5 / EN 16307-5 (accredited lab) | 4 | Accredited lab | On change | 10 yr |
| 6 | Chemical test report (PAH, phthalates, SCCP, SVHC) | 4 | Accredited lab | 2 yr | 10 yr |
| 7 | Technical file (MD Annex VII A / MR Annex IV) | 4–6 | Manufacturer | Ongoing | 10 yr |
| 8 | FPC plan and end-of-line load-test records | 5 | Supplier | On change / per lot | 10 yr |
| 9 | QMS / CSR audit reports (ISO 9001, BSCI) | 5 | QS / 3rd party | Per certificate | 5 yr |
| 10 | Sample validation, DPI, FRI reports (Annex A) | 5, 8 | QC / 3rd party | Per order | 5 yr |
| 11 | Golden sample record | 5 | QS | Per order | 5 yr |
| 12 | EC / EU Declaration of Conformity | 6 | Manufacturer | On change / legal act | 10 yr |
| 13 | EU economic operator designation (2019/1020 Art. 4) | 6–7 | Importer / AR | On change | 10 yr |
| 14 | Artwork, data plate and label approval | 7 | QS | Per revision | 5 yr |
| 15 | Instruction manual assessment (EN IEC/IEEE 82079-1) | 7 | QS | Per revision | 5 yr |
| 16 | Export packaging compliance + PPWR data record | 7–8 | QS / Merch. | Per shipment | 5 yr |
| 17 | SVHC declaration | All | Supplier | Per shipment | 10 yr |
| 18 | Shipment Release Certificate (SRC) | 8 | QS | Per shipment | 5 yr |

# 11. RACI Matrix

| Link | Responsible | Accountable | Consulted | Informed |
| --- | --- | --- | --- | --- |
| 1 — 4-Step determination | QS | QS Manager | Merchandiser, Ally | IQA, PPS-P |
| 2 — Pathway assignment | QS | QS Manager | Ally | PPS-P, Supplier |
| 3 — Module | QS | QS Manager | Accredited lab | Supplier, Ally |
| 4 — Type test / technical file | Supplier + lab | QS | QS | Ally, PPS-P |
| 5 — FPC / inspection | Supplier (FPC) + QC | QS | IQA, 3rd party | PPS-P, Ally |
| 6 — DoC | Manufacturer | QS | QS (review) | Ally, PPS-L |
| 7 — CE / marking / instructions | Supplier | QC (verify at FRI) | QS | Ally |
| 8 — SRC release | QS | QS Manager | QC | PPS-L, Ally, Merchandiser |

# 12. Regulatory Watch

| Date | Change | Impact on this category |
| --- | --- | --- |
| 20/01/2027 | Machinery Regulation (EU) 2023/1230 applies; MD 2006/42/EC repealed | New DoC (Annex V), importer marking, Annex III EHSR, instructions rules; re-issue DoC and technical file; update Annex A references. |
| Ongoing | OJEU citation of EN ISO 3691-5 / EN 16307-5 under MR | Presumption of conformity under MR depends on citation; monitor and document gaps. |
| 12/08/2026 → 2030 | PPWR (EU) 2025/40 staged obligations | Applies since 12/08/2026; harmonised sorting labels (Art. 12) and recyclability grades / empty-space ratio (Art. 24) in later phases — verify dates. |
| Jan / Jun yearly | REACH Candidate List updates | Refresh SVHC declarations per shipment. |
| Watch | Swiss MaschV revision aligning with MR | CH documentation after 20/01/2027. |
| In force | GPSR (EU) 2023/988 (since 13/12/2024) | Accident notification via Safety Business Gateway; online listing information. |

# 13. Companion Skills Reference

| Link | Deliverable | Companion skill |
| --- | --- | --- |
| 1–2 | QCP Storage/Moving; hEN confirmation | quality-compliance-pathway · harmonised-standard-lookup |
| 4 | Machinery risk assessment / DFMEA; test matrix | machinery-risk-assessment · fmea-generator · essential-characteristics-testing |
| 4–6 | Technical file | technical-construction-file |
| 5 | FPC; supplier audit; QC pack | factory-production-control · supplier-production-line-audit · qc-pack-excel-generator |
| 6 | QSD; DoC | qsd-generator · ce-marking-doc-dop |
| 7 | Artwork/data plate; instructions; packaging | artwork-packaging-compliance-master · assembly-instruction-assessment · export-packaging-compliance · ppwr-doc-generator |
| 8 | FRI validation; SRC | fri-validation · qc-pack-excel-generator |

# 14. Change Log versus Source Checklist

| # | Source item | Finding | Correction applied |
| --- | --- | --- | --- |
| 1 | General Product Safety Directive 2001/95/EC | Repealed 13/12/2024 | Replaced by GPSR (EU) 2023/988 (complementary to MD/MR). |
| 2 | Packaging Directive 94/62/EC, heavy metals ≤ 100 mg/kg | Replaced by PPWR (EU) 2025/40 from 12/08/2026 | PPWR Art. 5(4) limit retained; EPR and packaging data added. |
| 3 | Machinery Directive 2006/42/EC only | MR applies 20/01/2027 | Dual MD/MR references; switch-over rule by EU placing-on-market date. |
| 4 | EN ISO 3691-5 / EN 16307-5 without edition | Edition and amendment missing | EN ISO 3691-5:2015+A1:2020, EN 16307-5:2013 cited; verify OJEU. |
| 5 | Static test 1.5 × Q only | Dynamic test missing (MD 4.1.2.3) | Dynamic test 1.1 × Q added. |
| 6 | Pressure relief valve — text only | No test method | Overload attempt at 1.15–1.20 × Q added; sealing check. |
| 7 | No load holding / lowering control test | Gap | Drift and controlled-lowering test added. |
| 8 | No crushing / shearing point check | MD 1.3.7 gap | EN ISO 13854 gap check added. |
| 9 | Artwork check generic | Data plate / rated load / importer not listed | Mandatory marking elements with Critical classification added. |
| 10 | No sampling plan | Gap | ISO 2859-1 GI-II, Cr 0 / Ma 2.5 / Mi 4.0; S-3 for functional tests. |
| 11 | Defect table lacks functional defects | Gap | Oil leak, drift, lifting failure, missing CE/data plate, wrong-language manual added (Annex B). |
| 12 | Measurement letters (D/d, H, h, e, s, l, c, W, L, HH) undefined | Ambiguous | Parameter legend aligned with VDI 2198 type sheet (Annex C). |
| 13 | Coating adhesion method | No standard cited | ISO 2409 cross-cut reference; 15 % = class 2. |
| 14 | “CE certification” wording | Misleading — no NB certificate under Module A | Replaced by “DoC + accredited test report”. |

# Annex A — Sample Validation / Inspection Checklist (updated)

## A.1 General information

| Inspection type | ☐ Sample validation ☐ DPI ☐ FRI Re-inspection: ☐ No ☐ Yes |
| --- | --- |
| Reference | Packaging guideline for hardline; Offer; MCG-DIY-HT-PT-001 |
| Ally / Supplier / Factory | [___] / [___] / [___] |
| Inspection location / date | [___] / dd/mm/yyyy |
| Legal act applied | ☐ MD 2006/42/EC (EU arrival ≤ 19/01/2027) ☐ MR (EU) 2023/1230 (≥ 20/01/2027) |
| Documents / QC pack | Inspection checklist · PO/PI · Offer sheet · Shipping mark · Artwork (data plate, labels, warnings) · Instructions (all languages) · DoC · Test report · SVHC declaration · Golden sample record |
| Product | Ally ref [___] · Supplier ref [___] · Capacity Q [___] kg · Load centre c [___] mm · Fork length [___] mm · Qty [___] |
| Inspector / Reviewer | [___] / [___] |

## A.2 Assessment summary

| Criteria | Requirement | Evidence | Assessment |
| --- | --- | --- | --- |
| Factory QMS | QMS audit A/B (or ISO 9001); FPC audit | Audit report / certificate |  |
| Factory CSR | BSCI A/B/C (or SA8000); ISO 14001 if applicable | Audit report / certificate |  |
| EU Regulation | REACH (EC) 1907/2006 Art. 33, Annex XVII 50/51; POPs (EU) 2019/1021 | SVHC declaration + chemical test report |  |
| Machinery | MD 2006/42/EC → MR (EU) 2023/1230 | Signed DoC; technical file index |  |
| Product safety | GPSR (EU) 2023/988 (complementary); Reg. (EU) 2019/1020 Art. 4 | EU operator identification |  |
| Packaging | PPWR (EU) 2025/40 (Pb+Cd+Hg+Cr(VI) ≤ 100 mg/kg); FR Law 2020-105 Art. 112 (mineral oils) | Packaging test / declaration |  |
| Harmonised standards | EN ISO 3691-5:2015+A1:2020; EN 16307-5:2013; EN ISO 12100:2010 | Accredited test report |  |

## A.3 Defect definitions and sampling

- Critical (Cr): likely to result in a hazardous or unsafe condition, or non-compliance with a mandatory regulation.

- Major (Ma): likely to result in product failure or customer return; reduces usability or saleability.

- Minor (Mi): does not reduce usability; noticed on careful examination; may reduce saleability.

- Zone A = high visibility/touch in normal use; Zone B = partial; Zone C = none. Visual defect: Minor if visible at 30 cm, Major if visible at 100 cm.

- FRI sampling: ISO 2859-1 / ANSI Z1.4, General Inspection Level II; AQL Cr 0 · Ma 2.5 · Mi 4.0. On-site functional tests (A.4 item 7): Special Level S-3, Ac 0. Static test: sample validation and DPI only (1 pc).

## A.4 Inspection details

| # | Criteria | Method / requirement | Ref. | Fail class | Result |
| --- | --- | --- | --- | --- | --- |
| 1 | Quantity | DPI: ≥ 20 % produced. FRI: 100 % produced and ≥ 80 % packed. | A.R.E.N.A. | — |  |
| 2 | Packing | Method and condition; no damage, mould, infestation or odour; parts, tools, accessories per packing list; materials (poly bag, carton layers/thickness). Wooden pallet HT + IPPC mark (MB not accepted). | Packaging guideline; ISPM 15; PPWR | Mi–Ma |  |
| 3 | Artwork and marking | Compare with approved artwork: shipping mark; data plate (Section 8.1 elements); CE; rated capacity; importer/EU operator; warnings; instructions in all languages; DoC; Triman (FR); barcode clean, positioned and scanning correctly. | MD/MR 1.7.3, 1.7.4, 4.3.3 | Cr / Ma |  |
| 4 | Sample comparison | Conforms to golden sample and test report: model, capacity, fork dimensions, materials, pump type, wheel material, colour. | Test report; QSD | Ma |  |
| 5 | Measurement | Per PO/spec, package claims and Annex C; product tolerance table A.5; weight ± 5 %; packing dimensions and weight ± 5 %. | Spec | Pending / Ma |  |
| 6 | Workmanship | No scratch, flash, dent, crack, stain on visible surfaces; parts properly assembled; screws/nuts/rivets/pins/circlips secure; no missing or wrong accessories; no colour deviation; no sharp points/edges, rust; welds free of cracks, porosity, spatter; no hydraulic oil leakage. | MD/MR 1.3.4 | Per Annex B |  |
| 7.1 | Assembly | Assemble tiller step by step per instructions (pin, chain/rod adjustment). Difficult = Major; impossible or malfunction = Critical. | MD/MR 1.7.4 | Ma / Cr |  |
| 7.2 | Control system | Lift by pumping tiller; neutral position holds; lower by hand lever (and foot pedal if fitted); controls identifiable and return correctly; parking brake locks (if fitted). | MD/MR 1.2.1, 1.2.2 | Cr |  |
| 7.3 | Edges and surfaces | Tactile check of all accessible parts: no sharp edges, angles, rough surfaces. | MD/MR 1.3.4 | Cr |  |
| 7.4 | Tiller | Closed-loop handle or lateral hand protection; grip Ø within 25–35 mm (caliper); span ≥ 120 mm per hand. | EN ISO 3691-5 Cl. 4.2.2 | Cr |  |
| 7.5 | Functional verification (Q) | With rated load at c: lift to max height h3; travel ≥ 10 m on level floor; steer full lock both sides; lower controlled, no jerk/free fall. | EN ISO 3691-5 Cl. 5.2 | Cr |  |
| 7.6 | Load holding | Rated load raised to h3, hold 10 min [verify duration/limit per standard]; record drift; drift within limit. | EN ISO 3691-5 | Cr |  |
| 7.7 | Overload device | Attempt lift at 1.15–1.20 × Q: lifting prevented (where fitted); valve sealed / tool required. | EN ISO 3691-5 Cl. 4.4.3.3 | Cr |  |
| 7.8 | Static test (SV/DPI only) | 1.5 × Q centred at c, forks raised, 10 min [verify]; measure fork-tip height before/after: no permanent deformation, crack or leak. | MD/MR 4.1.2.3, 4.1.3 | Cr |  |
| 7.9 | Dynamic test | 1.1 × Q: 3 full lift/lower cycles + travel and steering; all functions operate correctly. | MD/MR 4.1.2.3, 4.1.3 | Cr |  |
| 7.10 | Crushing / shearing points | Gaps at tiller pivot, pump linkage, fork rollers, steer wheels: finger gap ≥ 25 mm or guarded; feet protection at steer wheels. | MD/MR 1.3.7; EN ISO 13854 | Cr |  |
| 7.11 | Leak check | After 7.5–7.9: no oil on pump, ram or floor. | FPC | Ma (Cr if lowering affected) |  |
| 7.12 | Smell check | At ≈ 10 cm: no abnormal odour. | A.R.E.N.A. | Ma |  |
| 7.13 | Marking rub test | 15 s water-soaked cloth, then 15 s with alcohol or hexane: marking remains legible, not easily removable, no curling. | IEC 60335-1 Cl. 7.14 method | Ma |  |
| 7.14 | Coating adhesion | Metal: 6 cuts, 2 mm spacing, cross-hatch, 3M 610/810 tape pressed 1 min, removed quickly: ≤ 15 % detached (ISO 2409 class ≤ 2). Non-metal: tape only, 0 % detached. | ISO 2409 | Ma |  |
| 7.15 | Coating thickness | Magnetic gauge, 5 points on frame and forks: DFT ≥ [___] µm per QSD. | ISO 2808 | Ma |  |

## A.5 Dimensional tolerances (from source)

| Product size (mm) | 0.5–3 | >3–6 | >6–30 | >30–120 | >120–400 | >400–1000 | >1000–2000 | >2000–3000 | >3000–5000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tolerance (mm) | ±0.1 | ±0.3 | ±1 | ±2 | ±5 | ±10 | ±20 | ±30 | ±50 |

| Packing size (mm) | ≤120 | >120–400 | >400–1000 | >1000–2000 | >2000–3000 | >3000–5000 |
| --- | --- | --- | --- | --- | --- | --- |
| Tolerance (mm) | ±5 | ±10 | ±20 | ±30 | ±50 | ±80 |

Remarks #1: [___]

# Annex B — Defect Classification

| Defect name | Zone A | Zone B | Zone C |
| --- | --- | --- | --- |
| Poor packing / Đóng gói kém | Minor – Major |  |  |
| Incorrect quantity / Sai số lượng đóng gói | Major |  |  |
| Incorrect packaging artworks / Sai thông tin in ấn đóng gói | Major |  |  |
| Missing packaging artworks / Thiếu thông tin in ấn đóng gói | Major |  |  |
| Poor label / sticker | Major |  |  |
| Rubbing product marking | Major |  |  |
| Sharp point / Điểm bén nhọn | Critical |  |  |
| Sharp edge / Cạnh bén nhọn | Critical |  |  |
| Shear and squeeze point / Điểm cắt và chèn ép | Critical |  |  |
| Rust / Rỉ sét | Major | Minor |  |
| Poor finishing / Hoàn thiện xấu | Minor – Major |  |  |
| Stain / Bị dính sơn, vết ố | Minor – Major | Minor |  |
| Over painting / Sơn chảy | Minor – Major | Minor |  |
| Coating peel off / Bong tróc sơn | Minor – Major | Minor |  |
| Blister / Bị phồng rộp | Minor – Major | Minor |  |
| Spots / Vết đốm | Minor – Major |  |  |
| Discoloration / Phai màu | Minor – Major |  |  |
| Dirty mark / Dấu vết bẩn | Minor – Major |  |  |
| Poor welding / Mối hàn tệ | Major | Minor – Major | Minor |
| Burr / Gờ chưa mài giũa | Minor – Major |  |  |
| Rough surface / Bề mặt thô ráp | Minor – Major |  |  |
| Uneven surface / Bề mặt cao thấp không đều | Minor – Major |  |  |
| Deformed / Bị biến dạng | Minor – Major | Minor |  |
| Damaged / Bị hư hỏng | Minor – Major | Minor |  |
| Scratched / Bị trầy xước | Minor | Accept | Accept |
| Broken / Bị gãy | Minor – Major | Minor |  |
| Chipped / Bị sứt mẻ | Minor – Major | Minor |  |
| Cracked / Bị nứt | Minor – Major | Minor |  |
| Dent mark / Dấu bị cấn móp | Minor – Major |  |  |
| Warpage / Cong vênh | Minor – Major | Minor |  |
| Wobble / Lung lay, gập gềnh | Minor – Major |  |  |
| Unfitting / Không vừa vặn | Minor – Major |  |  |
| Loose / Lỏng lẻo | Minor – Major |  |  |
| Over tolerance | Pending |  |  |
| Incorrect material / Sai nguyên vật liệu | Major |  |  |
| Incorrect colour / Sai màu sắc | Major |  |  |
| Incorrect specification / Sai thông số kỹ thuật | Major |  |  |
| Difficult assembly / Lắp ráp khó | Major |  |  |
| Impossible assembly / Không lắp ráp được | Critical |  |  |
| Difficult folding / Gấp xếp khó | Major |  |  |
| Difficult function / Chức năng sử dụng khó | Major |  |  |
| Fail coating rubbing / adhesion test | Major |  |  |
| NEW Cracked or poor weld on load-bearing frame / fork / Mối hàn chịu lực nứt | Critical |  |  |
| NEW Cannot lift rated load / Không nâng được tải định mức | Critical |  |  |
| NEW Uncontrolled lowering or drift beyond limit / Hạ tải không kiểm soát | Critical |  |  |
| NEW Permanent deformation after static test / Biến dạng vĩnh viễn | Critical |  |  |
| NEW Overload device missing / not sealed (where required) / Thiếu van quá tải | Critical |  |  |
| NEW Hydraulic oil leakage / Rò rỉ dầu thủy lực | Major (Critical if lowering affected) |  |  |
| NEW Parking brake ineffective (if fitted) / Phanh không hiệu quả | Critical |  |  |
| NEW Missing / incorrect CE marking or data plate / Thiếu nhãn CE, nhãn thông số | Critical |  |  |
| NEW Rated capacity missing or wrong / Thiếu hoặc sai tải định mức | Critical |  |  |
| NEW Instructions missing or not in required language / Thiếu HDSD hoặc sai ngôn ngữ | Critical |  |  |
| NEW Importer / EU operator identification missing / Thiếu thông tin nhà nhập khẩu | Critical |  |  |
| NEW Missing / loose circlip, pin or axle fastener / Thiếu, lỏng khóa chốt | Critical |  |  |
| NEW Wheel not rotating freely / Bánh xe quay không trơn | Major |  |  |
| NEW Wooden pallet without ISPM 15 mark / Pallet gỗ không có dấu IPPC | Major |  |  |

Reference defect photos: retain the source photo set (poor finishing, poor welding, scratched, coating peel off, rough surface, broken, dent mark) in the QC pack.

# Annex C — Measurement Data Sheet

Parameter symbols aligned with the VDI 2198 type sheet for industrial trucks (replaces undefined source letters D/d, H, h, e, s, l, c, W, L, HH).

| Symbol | Parameter | Specification | Claimed on package | Finding | Requirement | Result |
| --- | --- | --- | --- | --- | --- | --- |
| Q | Rated capacity (kg) |  |  |  | Matches data plate & test report |  |
| c | Load centre distance (mm) |  |  |  | Matches data plate |  |
| h3 | Lift height (mm) |  |  |  | Within tolerance |  |
| h13 | Fork height, lowered (mm) |  |  |  | Within tolerance |  |
| h14 | Tiller height in operating position (mm) |  |  |  | Within tolerance |  |
| l1 / l2 | Overall length / length to fork face (mm) |  |  |  | Within tolerance |  |
| b1 / b5 | Overall width / width across forks (mm) |  |  |  | Within tolerance |  |
| s / e / l | Fork thickness / width / length (mm) |  |  |  | Within tolerance |  |
| D1 / D2 | Steer wheel / load roller diameter (mm) |  |  |  | Within tolerance |  |
| — | Grip Ø / span (mm) |  |  |  | 25–35 / ≥ 120 |  |
| — | Unladen mass (kg) |  |  |  | ± 5 %; matches data plate |  |
| — | Packing dimensions (mm) |  |  |  | Conform to package claim |  |
| — | Packing gross weight (kg) |  |  |  | ± 5 % |  |
| — | Barcode |  |  |  | Correct with spec & print |  |

# Annex D — Packaging Disposal Data (PPWR)

Required for every SKU: ☐ N/A ☐ Pending ☐ Yes. Weight in grams per material.

| Level | Units / pack | Paper, cardboard | Plastic foil / bags | Plastic straps | Metal | EPS | Wood |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Primary (sales unit) |  |  |  |  |  |  |  |
| Transport unit |  |  |  |  |  |  |  |

Batteries / lamps / EEE table: N/A (manual truck without electrical equipment). Re-assess if a scale or electric lift variant is added.